A directional Helical aerial for HRPT reception

Introduction

This article describes the design and construction of a circular helical aerial of 1.5 meter length.    This antenna is suitable only for the reception of digital HRPT signals such as those transmitted by the NOAA satellites on 1698 and 1707 MHz.

The aerial described is experimental,    however with a good low noise pre-amplifier,    which is to be described later,      it is possible to decode noise free NOAA signals from 20o elevation (tested with a properly functioning LNC 1700 converter and the HPR 137 receiver).

Reception using a 90 cm or bigger dish will be better if    such a parabolic aerial can be mounted on an az/el rotator.    Such an arrangement however requires heavy duty az/el rotators and these are rather expensive.    Furthermore a large dish catches a lot of wind,    but as an alternative chicken wire could be used.

The helical to be described weighs less than 1.5 kg (with the LNA attached) and can be mounted on two used TV aerial rotors (Channelmaster/Stolle).

The required receiver and decoder were discussed in an earlier “Kunstmaan” article.    The German DJ9BV LNA (LNAH-1.3 or LNAH-1.7) kit was used.

Theory

The HRPT signal is transmitted with RH circular polarization (RHCP).    The same polarization is used for APT transmissions in the 137 MHz band.    This mode of polarization has been chosen,    because linear polarization (vertical or horizontal as is the case with Meteosat),    is less suitable for polar orbiting satellites.    During the pass of a polar orbiting satellite,    the angle of the satellite in relation to the atmosphere and the distance of the satellite to the receiving aerial changes continuously.    A horizontally polarized signal will suffer scattering through the atmosphere and would also show a certain amount of rotation at the time it arrives at the receiver.    Such a signal would additionally be reflected by certain obstacles on earth.    When the aerial receives both the reflected signal as well as the direct signal,    it is possible that the direct signal is attenuated by the reflections and sometimes may even be completely eliminated.

These problems do not occur when circular polarization is used and rotation through atmospheric influences does not affect the signal.    When a circularly polarized signal is reflected the direction of rotation changes.    A RHCP signal is transformed into a LHCP signal through reflection and the receiving aerial (designed for RHCP),    receives the signal at a considerably attenuated level,    since it is insensitive to LHCP signals.

Although a circularly polarized signal can be received with an horizontal or vertical aerial,    this would however result in a 3 dB signal loss (half of the signal would not be received).    Experience has shown,    that the signal strength not only varies considerably,    but sometimes it even disappears completely for a few seconds when the satellite is at a particular elevation.

The reverse is equally true;    the helical is not suitable for Meteosat reception,    since the horizontally polarized signal from Meteosat would be received with a loss of 3 dB.

It is also possible to use two yagis mounted at right angles to each other (crossed yagi configuration).    As with the crossed dipole aerial this would require a phase delay line and impedance matching arrangement,    which is rather difficult to construct for 1700 MHz.

This is the reason why the helical aerial was chosen.    Many satellites too employ this type of aerial,      for transmission of signals to earth.

The helical consists of an element,    which is wound either in a clock-wise or anti clock-wise direction,    with the signal taken from the end of the spiral at the reflector side.

Advantages of the helical:

+	exact circular polarization

+	less weight as compared to a dish

+	less wind loading

+	simpler as compared to a crossed yagi

Disadvantages of the helical:

-	at this stage experimental and not quite optimal.    There are many calculation 	programmes as well as proven designs for yagis,    information on helicals however is 	scarce.

-	less gain as compared to a dish aerial or crossed yagi of the same length (crossed yagi 	of 1.5 m results in 0.8 dB or more gain).

-	an impedance matching transformer is required.    The helical has a high characteristic 	impedance of 140 Ohm at the feed point,    which has to be converted effectively to 50 	Ohm for the LNA.

Construction of the helical

The helical consists of a spiral of 35 turns with a diameter of 60 mm (center to center).    The distance between turns is 42.8 mm (center to center.),    which corresponds to a pitch-angle of 12.8 degrees.    Nickel plated copper wire with a diameter of 2.76 mm (earthing wire 6 mm2) was used for the spiral.

The turns are kept in their correct position by means of plastic supports and the inner insulation of RG 213 coax was used for this purpose, as this is a good insulator and is easy to work with.

In order to produce a sturdy assembly, a support is fitted every 1 3/4 turn, plus an additional one at the end.

The last turn at the end should be of a smaller diameter, i.e. the end of the helical should be conical, for which purpose the last support is a little shorter.

The support mast is a 15 mm square tube, of 1 mm thickness.    The reflector is constructed from 2 mm aluminium sheet and is bolted to the main support with two pieces of angle profile aluminium.

A floating strip is mounted above the reflector, which forms the 1/4  impedance match, which transforms the impedance of the helical to 50 Ohms for the LNA.    To provide support and to facilitate fine tuning of the impedance match, a nylon adjustment screw is fitted between the strip and the reflector.

Because the helical is an open aerial, an earthing loop has to be provided from the N connector to earth.    This prevents static interference and avoids damage to the LNA.

The LNA is connected directly to the N-connector on the rear side of the reflector.

The helical can be fitted to a rotor by means of a plastic pipe in the middle of the support mast and a mounting bracket, with a 40 mm plastic tube fitting exactly between the spiral.

Constructing the helical

The copper wire should be free from kinks.    Clamp one end of the wire in a vice and firmly pull a cloth over the wire away from the vice (stretching, straightening of the wire).

Wind the wire, with closely spaced turns on a mandrel (pipe or wooden dowel) of approx. 55 mm diameter.    MAKE SURE THAT YOU ARE WINDING IN THE RIGHT DIRECTION !! (refer to 3D View in the drawing)

When winding is completed the spiral will spring back to a larger diameter.

File an angle of approx. 13 degrees at one end, which later will be soldered directly to the impedance matching strip.

Whilst the spiral is still on the mandrel pull the turns evenly away until the center to center distance of each turn is about 43 mm.

Now mark the distances on the 15 X 1 mm support beam and use a square to continue the markings all around the support.    A suitable mark should also be made for the reflector.

With the aid of the 3D view on the drawing, determine where every 1 3/4 turn supports are to be placed.    Drill the holes completely through the square tube, first with a 3 mm drill, using a drill stand, followed by enlarging the holes on one side of the square tube with a 7 mm drill in those places where a support is to be mounted.    Ensure that you drill only one side and only in those places where a support will be inserted !

Use a piece of 7 mm inside diameter pipe to make a drill/cutter mandrel, to cut the supports to the correct length (refer drill/cutter drawing).

Cut off about 1 meter of RG 213 coax and slice the black outer insulation open over the length of the piece, followed by removing the insulating layer.    Strip off the copper braid.    The most difficult part is to pull out the twisted multiple wire inside conductor.    It may be helpful to cut the 1 meter length into 2 or 3 equal lengths, followed by pulling the inner conductors out with a pair of pliers.    If the process proves to be excessively difficult, the lengths of wire may be soaked in hot water for a few minutes.

With this operation successfully completed, the 21 supports can be cut to size with a sharp knife,    next to the end of the drill/cutter.    Try and cut the ends as straight as possible.

The 21st support has to be cut a little shorter, as this is the last one (refer drawing).

Now drill a 3 mm hole in each one of the supports, whilst they are still inside the drill/cutter (place the drill/cutter length-wise in a vice, to ensure that the supports are clamped tight).

Prepare the reflector sheet and supports as per drawing and drill the required holes for the connector and supports.    Make sure that the connector fits tightly in the hole and that the flange lies flat on the reflector surface !    All connector fitting holes should be counter-sunk, with the exception of one.    Drill a 2.5 mm hole and tap this with M3 thread for the nylon adjustment screw.    If a M3 tap is not available, a matching nut may be glued into position at the other end of the reflector.    The impedance matching strip-line can be cut out of 1 mm copper or brass sheet with a jig or coping saw.

Assembly

Slide the supports onto the spiral one by one and distribute them evenly over the complete length.    Do not as yet fit the short support piece.

Slide the ready drilled support boom in the spiral and push the supports into the corresponding holes in the square tube one by one.    Note that a half turn protrudes past the last support at the reflector side.

Lastly fit the short support piece at the start.    To do this, the first turn will have to squeezed a little smaller.    Attach all supports with a self tapping screw.

Distribute the turns evenly by sliding through the supports and check that all turns are of the same diameter and are properly in line, starting from the front.    This is a tedious exercise, but for good results, it is essential that there is no more than a maximum of 3 mm difference in the diameter or positioning of the windings.

The N-connector and reflector supports are now fitted to the reflector.    Slide the reflector onto the support boom and attach it in the right place.

Push the matching strip over the connector pin.    Use a sheet of wood or cardboard of approx. 3.6 mm thickness to temporarily position the matching strip at the correct distance from the reflector.    Solder the end of the spiral to the strip and the strip to the connector pin..    Fasten the earthing loop in place and solder this to the matching strip against the connector pin.

The LNA is screwed directly to the N-connector at the other side of the reflector.

Adjustment and testing of the helical

Point the aerial toward Meteosat.    Reception will not be noise free.    Turn the nylon screw several turns either way and determine from the APT image in which position the least number of speckles appear in the image.

Changing the position of the screw changes the impedance as seen by the LNA and it is possible that hardly any change will be observed, depending on the type of LNA.    Theoretically the impedance will be 50 Ohms, when the full length of the impedance matching strip floats 3.6 mm evenly above the surface of the reflector.

Note that when the first 4-5 turns near the connector are touched, the signal is strongly attenuated.    It is necessary to have an obstruction free view of 185 degrees in the azimuth plane and approx. 30 degrees in elevation.

When Meteosat can be reasonably well received, it is possible to attempt following a NOAA pass by hand.    Accurate pointing is not as critical as compared to a dish because even when the aerial is pointed 10 degrees off target, half of the signal strength should remain.

Finishing the helical

When proper functioning of the aerial has been confirmed and all the turns are circular and evenly aligned, the open ends of all supports are filled in with silicone resin.    Ensure that you apply some pressure when squirting in the hole, to ensure that both spiral an support are covered with silicone,    as no water should be allowed to enter between spiral and support.    The point of contact between support and square support boom must also be covered in silicone, as well as each of the self-tapper heads.

Close both ends of the support off with a stopper or some silicone resin.

The flange where the N-connector seats on the reflector must be waterproofed all-round with silicone, the same applies to bolt heads and nuts (at the rear of the reflector).

The point where the impedance matching strip is attached to the N-connector pin and the space to the reflector surface must filled with silicone.    Take care not to apply too much silicone between the matching strip and reflector surface - only the area immediately around the N-connector is to be filled.    Use transparent silicone resin and leave the assembly for at least 2 days, to ensure complete curing of the resin, since incompletely cured silicone results in considerable signal losses !

Weathering

The first turns and the matching strip are sensitive to touching, hence direct cables etc. as far as possible away from the spiral.    The matching strip is affected by rain which might accumulate between the reflector and strip.    A temporary solution would be to spray this area with silicone-spray.    It is however better to cover the first 4-5 turns with a transparent plastic cup.    Note that coloured plastics often contain heavy metal pigments (cadmium yellow,    zinc oxide white and titanium dioxide),    which could cause severe signal attenuation !

Experimenting with the helical

Calculations have shown, that greater signal gain and narrower band-width can be obtained, when the turns are closer together (e.g. 10 degrees instead of 12.8).    This design is characterized by wide band-width, which is not required for HRPT.    Unfortunately the formulae found in reference books are not entirely clear regarding the optimum angle.    According to the formula, an angle of 2 degrees should produce super signal gain, however do not try this !

Theoretically the signal strength may be doubled by placing two aerials next to each other, but the distance between them and the method used to combine the two signals is critical.    To do this 100 Ohm coax and a different impedance matching strip are required.    To increase the length of the aerial beyond 1.5 m (more than 35 turns) has little effect and two aerials of 1 meter length provide more gain as compared to one aerial of 2 meters !

The principle of a helical can also be used to produce a feed which is placed at the focal point of a parabolic dish.    In this case the helix has to be wound anti-clockwise (reflected signal from the dish inverts the direction !) and should have no more than 6 turns,    of somewhat different dimensions.    When combined with a 90 cm dish constructed of chicken wire this arrangement produces good results as an HRPT receiving aerial.    The author is planning to carry out further experiments and tests with such a design.

Bibliography

CONNECT> February 1995 (The magazine for packet radio working group the Netherlands)

IEEE volume AP-28, No 2 March 1980 + volume 30 No. 5 May 1990

Rothammel and Satellite experimenters handbook

Electronic & Wireless World, June 1985

UKW-BERICHTE 3/83 and an article by Bob Atkins (KA1GT)

Internet:		TACO communications

SPIRIT RIVER Pty., Ltd.

With thanks to R.W. Hollander for his research and Chris van Lint for the translation.

Arne van Belle

Coordinator Radio Observers Work Group Kunstmanen

Calculations for a helical aerial for HRPT-reception:

Design frequency:	 MHz 		v-light	    km/s

Wave length:						mm

Element-thickness:				      mm

circumference helical:				mm

Diameter winding:					 mm (centre to centre)

No of turns:					pitch-angle	          degrees

Pitch of windings:			          mm (centre to centre.)

Total length spiral:					mm

Maximum achievable gain:	                     dBi      (equals approx. 15.7 dBd)

Band-width ( -0.5dB border):

MHz

Beam-width (-3dB):						degrees

Length of stretched spiral:				mm

Impedance of helical:					 Ohm

Impedance of balun:					 Ohm

(1/4 wave stripline above GND to 50 Ohm)

Dimensions of stripline:          	length = 44 mm	      width = 8 mm        distance to GND = 3.6 mm

Note: 	angles to be entered as radials!

dBi = gain compared to isotropic radiator (theoretical spherical -form), dBd is gain compared to a dipole